Module 6 Assignment

Krishna M

Mepco Schlenk Engineering College, Sivakasi

- Capture and analyze ARP packets using Wireshark. Inspect the ARP request and reply frames when your device attempts to find the router's MAC address.

- Manually configure static routes on a router to direct packets to different subnets. Use the ip route command and verify connectivity using ping and traceroute.

- Given a network address of 10.0.0.0/24, divide it into 4 equal subnets.Calculate the new subnet mask.Determine the valid host range for each subnet. Assign IP addresses to devices in Packet Tracer and verify connectivity.

Given Network: 10.0.0.0/24 (Subnet Mask: 255.255.255.0)

We need 4 subnets, so we borrow 2 bits from the host portion.

New Subnet Mask: /26 (255.255.255.192)

Each subnet now has:

64 IPs (2^6)

62 usable host IPs (excluding network and broadcast IPs)

Subnet	   Network Address	First Usable IP	Last Usable IP	Broadcast Address

Subnet 1	10.0.0.0/26	    10.0.0.1	    10.0.0.62	   10.0.0.63

Subnet 2	10.0.0.64/26	10.0.0.65	    10.0.0.126	   10.0.0.127

Subnet 3	10.0.0.128/26	10.0.0.129	    10.0.0.190	   10.0.0.191

Subnet 4	10.0.0.192/26	10.0.0.193	    10.0.0.254	   10.0.0.255

- You are given three IP addresses: 192.168.10.5, 172.20.15.1, and 8.8.8.8. Identify the class of each IP address. Determine if it is private or public. Explain how NAT would handle a private IP when accessing the internet.

IP Address	    Class	    Private/Public	IP Range for Class

192.168.10.5	Class C	    Private	        192.0.0.0 – 223.255.255.255

172.20.15.1	    Class B	    Private	        128.0.0.0 – 191.255.255.255

8.8.8.8	        Class A	    Public	        1.0.0.0 – 126.255.255.255

Private IP addresses (192.168.10.5 and 172.20.15.1) cannot be routed on the public internet.

Network Address Translation (NAT) allows devices with private IPs to communicate over the internet by converting private IPs into a single public IP address assigned by an ISP.

How NAT Works:

A device with a private IP e.g., 192.168.10.5 sends a request to access a public server e.g., 8.8.8.8.

The router performing NAT replaces the private source IP like 192.168.10.5 with a public IP e.g., 203.0.113.10.

The response from the public server is received by the router.

The router uses a NAT translation table to forward the response back to the correct internal device (192.168.10.5).

- In Cisco Packet Tracer, configure NAT on a router to allow internal devices (192.168.1.x) to access the internet.Test connectivity by pinging an external public IP. Capture the traffic in Wireshark and analyze the source IP before and after NAT translation.